# Functional Test for ESP32 Rainbow PCB Assembly

## Required Equipment

- USB-C cable

- USB Power supply

## Test Procedure

# Step 1: Connect Over USB-C

- Plug a USB-C cable into the PCB’s USB-C connector.

- Connect the other end of the cable to a computer.

# Step 2: Confirm Green LED D1 is Lit

1. Verify that green LED D3 (Power Indicator) is illuminated.

## Pass Criteria:

- D3 is ON → Proceed to Step 3

## Troubleshooting

- D3 is OFF → Check soldering on USB connector J1, ICs: U12, U2. Check test points 5v and PWR. Measured voltage should be 5v

# Step 3: Move Switch SW1 to Left Position

- Locate switch SW1 on the PCB.

- Move the switch to the left position.

# Step 4: Confirm Green LED D2 is Lit

- Verify that LED D2 is illuminated after moving the switch.

## Pass Criteria:

- D2 is ON → Passed

## Troubleshooting

- D2 is OFF → Check switch position. Check soldering on U6. Check test point 3V3

# Step 4 Confirm USB device is present on PC

USB device should appear on the PC.

## Pass Criteria:

- Device appears on the PC

### Troubleshooting

- Check soldering on USB connector

- Check soldering on ESP32

- Check soldering on U12